- BLINKING LED’S

int led =12;

void setup()

{

pinMode(led, OUTPUT);

}

void loop()

{

digitalWrite(led, HIGH);

delay(1000);

digitalWrite(led, LOW);

delay(1000);

}

EXP-2 PUSHBUTTON LED

int buttonstate =0;

void setup()

{

pinMode(6,INPUT);

pinMode(12, OUTPUT);

}

void loop()

{

buttonstate = digitalRead(6);

if (buttonstate == HIGH){

digitalWrite(12,HIGH);

} else {

digitalWrite(12,LOW);

}

delay(10);

}

EXP-3 TRAFFIC LIGHT SIMULATION

// C++ code

//

int x = 0;

long readUltrasonicDistance(int triggerPin, int echoPin)

{

pinMode(triggerPin, OUTPUT);  // Clear the trigger

digitalWrite(triggerPin, LOW);

delayMicroseconds(2);

// Sets the trigger pin to HIGH state for 10 microseconds

digitalWrite(triggerPin, HIGH);

delayMicroseconds(10);

digitalWrite(triggerPin, LOW);

pinMode(echoPin, INPUT);

// Reads the echo pin, and returns the sound wave travel time in microseconds

return pulseIn(echoPin, HIGH);

}

void setup()

{

pinMode(8, OUTPUT);

Serial.begin(9600);

pinMode(9, OUTPUT);

pinMode(10, OUTPUT);

}

void loop()

{

digitalWrite(8, LOW);

x = 0.01723 * readUltrasonicDistance(6, 5);

Serial.println(x);

if (x>=0 && x<=50)  {

digitalWrite(8, HIGH);

} else {

digitalWrite(8, LOW);

}

if (x>=51 && x<=75)  {

digitalWrite(9, HIGH);

} else {

digitalWrite(9, LOW);

}

if (x>=76) {

digitalWrite(10, HIGH);

} else {

digitalWrite(10, LOW);

}

delay(10); // Delay a little bit to improve simulation performance

}

EXP-4 BUZZER CIRCUIT

// C++ code

//

int push_button=12;

int buzzer=6;

int b=0;

void setup()

{

pinMode(push_button,INPUT);

pinMode(buzzer, OUTPUT);

}

void loop()

{

b=digitalRead(12);

if (b==HIGH){

digitalWrite(buzzer,HIGH);

} else{

digitalWrite(buzzer,LOW);

}

}

EXP-5 TEMPERATURE SENSOR AND LCD

#include <Wire.h>                     // Library for I2C communication

#include <Adafruit_LiquidCrystal.h>    // Library for the LCD

// Set the I2C address for the LCD (typically 0x27 or 0x3F)

Adafruit_LiquidCrystal lcd(0);

// TMP36 sensor is connected to analog pin A0

const int sensorPin = A0;

void setup() {

// Initialize the LCD

lcd.begin(16, 2);      // 16 columns, 2 rows

lcd.setBacklight(1);    // Turn on the LCD backlight

// Display welcome message

lcd.setCursor(0, 0);

lcd.print("Temp Sensor Init");

delay(2000); // Wait for 2 seconds

lcd.clear();

}

void loop() {

// Read the value from the TMP36 sensor

int sensorValue = analogRead(sensorPin);

// Convert the analog value to voltage (assuming 5V reference voltage)

float voltage = sensorValue * (5.0 / 1023.0);

// Convert the voltage to temperature in Celsius

float temperatureC = (voltage - 0.5) * 100.0;

// Convert temperature to Fahrenheit (optional)

float temperatureF = temperatureC * 9.0 / 5.0 + 32.0;

// Display temperature on the LCD

lcd.setCursor(0, 0);            // Set cursor to first row

lcd.print("Temp: ");

lcd.print(temperatureC);        // Display Celsius temperature

lcd.print(" C");

lcd.setCursor(0, 1);            // Set cursor to second row

lcd.print("Temp: ");

lcd.print(temperatureF);        // Display Fahrenheit temperature

lcd.print(" F");

delay(1000); // Update the temperature reading every second

}

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <DHT.h>

#define DHTPIN 2        // Pin where the DHT11 is connected

#define DHTTYPE DHT11   // Define the sensor type as DHT11

DHT dht(DHTPIN, DHTTYPE);

LiquidCrystal_I2C lcd(0x27, 16, 2); // Set the LCD I2C address

void setup() {

lcd.init(); // Initialize the LCD

lcd.backlight(); // Turn on the LCD backlight

dht.begin(); // Start the DHT sensor

}

void loop() {

float tempC = dht.readTemperature(); // Read temperature in Celsius

float tempF = (tempC * 9.0 / 5.0) + 32.0; // Convert to Fahrenheit

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Temp: ");

lcd.print(tempC);

lcd.print(" C");

lcd.setCursor(0, 1);

lcd.print("Temp: ");

lcd.print(tempF);

lcd.print(" F");

delay(2000); // Wait for 2 seconds before next update

}